|  | Palm Beach County Fire Rescue Standard Operating Guideline SOG 510-05 Blood Transfusion Issue Date: 05/12/2022 Effective Date: 05/12/2022 |
| --- | --- |

PURPOSE:

To establish guidelines for proper procurement, storage, maintenance and administration of blood products.

AUTHORITY:

- Fire Rescue Administrator

- Medical Director, Palm Beach County

SCOPE:

This standard operating guideline shall apply to Medical Services, all EMS Captains, and Flight Crews.

ATTACHMENTS:

Attachment A:		Blood Transfusion Report

Attachment B:		Whole Blood Daily Check-Off Form

Procedures:

These guidelines provide a basic framework for blood products.

- Definitions:

  - Blood Bank Refrigerator – Approved refrigeration device to store blood products long term.

  - Portable Blood Cooler – Approved cooler used to store blood outside of the blood refrigerator, and deliver products to the field.

  - TIC™ Freezer – Separately maintained freezer to hold the cooler insert panels (TICs).

  - Thermal Isolation Chambers (TIC™) Panels – TIC™ coolants filled with phase change material and vacuum insulated panels (VIP) keep medical materials at the required temperature for up to 48 hours.

  - Temp° Stick™️ – Temperature monitoring device placed within the blood Refrigerator and blood cooler.

  - Fluid Warmer – Approved device to be used during administration of whole blood to warm the product prior to infusing.

  - LTOWB+ – Low titer Group O+ Leukoreduced Whole Blood

  - Blood Transfusion Report (Attachment A) – Form from the blood bank that accompanies each unit of whole blood.  This form must be completed for each administration.

  - Whole Blood Daily Check-Off Form (Attachment B) – PBCFR form that requires each unit of blood be visually inspected and temperature logged twice daily.

- Procurement – Blood products are distributed through ONE BLOOD, via regular/requested delivery.

  - The Medical Services Division will serve as the centralized liaison to contact ONE BLOOD to order LTOWB+.

  - Leukoreduced O+ Whole Blood

  - LTOWB+, titer should be < 1:256

- Storage – Upon receipt of the blood products, each item will be logged on the Whole Blood Daily Check-Off Form and placed in the blood bank refrigerator at Battalion Headquarters or Trauma Hawk.

  - Ensure all tags and unit stickers are correct for (LTOWB+) and are affixed to the bags.

  - Ensure Temp Dot is affixed to blood bag and indicated “green”.

- Temperature Controls – The blood bank refrigerator and portable blood cooler will be visually inspected and temperature checked twice each day, at 0730 and 1930, and recorded on the Whole Blood Daily Check-Off Form. Each unit of whole blood in the possession of PBCFR or Trauma Hawk must be logged.

  - If at any time the affixed Temp Dot is indicated solid “red”, or the blood appears cloudy, discolored, or frozen, it is considered compromised and must be removed from service.

  - A Temp° Stick™️ shall be used to continuously monitor the temperature of any container used to store or transport whole blood (blood bank refrigerator and portable blood cooler).

  - Ensure the Temp° Stick™️ is connected to Wi-Fi as indicated on the Temp° Stick™️ app.

  - An alert will be sent to the cell phone assigned to the Unit in possession of the whole blood if the Temp° Stick™️ detects a temperature out of range (< 1 degree Celsius or  > 8 degrees Celsius).

  - If an alert is received from the Temp° Stick™️, the storage container and whole blood should be immediately inspected and the issue should be corrected if possible.

  - Visually inspect and log the temperature of each unit of whole blood stored in blood refrigerator and blood cooler twice daily (2-6 degrees Celsius).

  - Each unit of whole blood must be documented on its own individual daily check-off form and visually inspected twice daily while in storage/possession of PBCFR or Trauma Hawk.

  - Return and exchange each unit of whole blood to the Blood Bank or administer to a patient, on or before the day 21 possession date.

  - Forward a copy of all completed Whole Blood Daily Check-Off Forms to the Medical Services Division: Fire-MSD@pbcgov.org

- General Instructions:

  - Do not delay transport to ED to initiate blood products. Administration of blood products may be initiated as soon as indicated per “Whole Blood Transfusion” protocol.

  - Do not give medications through the Whole Blood IV/IO set.

    - Utilize alternate access for medication administrations via IV/IO while blood products are being administered.

- Portable Blood Cooler Usage – At the beginning of each shift, six TIC™ panels should be removed from the TIC™ freezer.

  - Allow TIC™ panels to thaw for 25 minutes in flat position.

  - The TIC™ panels should be wiped of any moisture/condensation and then loaded in the portable blood cooler.

  - The blood products bag(s) should be removed from the blood bank refrigerator and placed inside the cooler, assuring temperature controls are in place (Temp° Stick™️).

  - The routine time for products in this blood cooler is to be 24 hours, with a maximum time of 48 hours.

  - The TIC™ panels from the previous shift shall be rotated back to the TIC™ Freezer and allowed to condition for at least 24 hours at -18 degrees Celsius.

  - Be sure the temperature monitoring device (Temp° Stick™️) is maintained in the portable blood cooler and monitored throughout the shift.

- Rotation – 72 hours prior to expiration of LTOWB+, contact the Medical Services Division for replacement: Fire-MSD@pbcgov.org

  - The Medical Services Division will serve as the centralized liaison to contact ONE BLOOD to re-order LTOWB+.

  - Blood can be exchanged 21 days prior to expiration, with a 24-hour turn-around time on ordering.

- Blood Transfusion Report – Any time blood products are administered a Blood Transfusion Report shall be completed utilizing the form supplied by the blood bank. The following process shall be completed:

  - The Blood Transfusion Report is filled out

  - Remove barcode sticker from blood product bag and apply to both white and yellow copies of the Blood Transfusion Report.

  - Attach the receiving hospital’s patient identification label to both the white and yellow copy of the Blood Transfusion Report.

  - The incident number shall be written on the top of the Blood Transfusion Report.

  - The white copy of Blood Transfusion Report shall remain with the patient.

  - The yellow copy of Blood Transfusion Report shall be returned to the blood bank.

    - Scan and e-mail copy to the Medical Services Division: Fire-MSD@pbcgov.org

    - Medical Services will return the form to the blood bank.

    - Medical Services will electronically attach the form to the ePCR.

- EPCR Documentation – The following shall be documented within the EPCR:

  - Type, titer and volume

  - Reason for blood

  - Administration site

  - Patient’s temperature before, during and after administration

  - Response to blood products

  - Fluid warmer usage

  - Any adverse reactions

  - If verbal or implied consent was obtained regarding blood products

- Adverse Reaction – Immediately STOP, maintain alternate fluids and follow appropriate protocol. Any transfusion reaction, must be reported to the receiving facility.

  - Consistently watch for transfusion reaction.

  - If an anaphylactic reaction exists, stop transfusion, change all lines and refer to “Allergic Reaction” section of the EMS Patient Care Protocols.

  - The EMS Captain/Flight Crew shall ensure the blood products, with all tubing (LifeFlow tubing and QinFlow DU), will be packaged in a red bag and returned to the blood bank.

- Religious Observations – Some religions will refuse to accept blood products. In this instance follow EMS protocol, and document the refusal in the EPCR.

- Out of Temperature or Compromised – If any blood product is discovered out of temperature range (that has not been immediately corrected) and/or is visually compromised in any way:

  - If at any time the affixed Temp Dot is indicated solid “red”, or the blood appears cloudy, discolored, or frozen, it is considered compromised and must be removed from service.

  - Complete a Property and Liability Accident/Incident Report and obtain a Property Damage Tracking Number per FR-T-302

  - Take the blood out of service, store in the blood bank refrigerator and label “out of service”.

  - The blood bank will be contacted and an incident investigation will be done to prevent reoccurrence.

- QinFlow Portable Fluid Warmer – The fluid warmer should be used whenever possible during the administration of LTOWB+.  Routine care and operation should include:

  - Turn on the unit daily and ensure the battery level is adequate.

  - Batteries do not need to be depleted completely before recharging.

  - Batteries should be swapped weekly, every Monday, and recharged after each use.

  - The base unit, battery, and battery charger are reusable devices.

  - Clean the base unit and battery after each use.

    - Cleaning should consist of wiping the external covering of the base unit with a department approved anti-germicidal wipe.

    - Do not submerge, sterilize or autoclave the base unit or the battery.

Supersession History

- SOG #510-05, issued 05/12/2022